# STANDARD OPERATING PROCEDURES FOR HAZARDOUS AND PARTICULARLY HAZARDOUS CHEMICALS

For

2-Methyl-2-Butanol

| 1. PROCEDURE / PROCESS | 2-Methyl-2-butanol is used in Building, Room. Insert procedure here: |
| --- | --- |
| 2. CHEMICAL NAME(S) and associated PHYSICAL and HEALTH HAZARDS | 2-Methyl-2-butanol CAS# 75-85-4; also known as tert-Amyl alcohol; tert-Pentyl alcohol - Highly Flammable liquid and vapor. - Harmful if swallowed or if inhaled. - Causes skin irritation. May be absorbed through the skin. - May cause respiratory irritation. - May cause eye irritation. Signal Word: DANGER Toxicological Data: ORAL (LD50): 1,000 mg/kg [Rat] *Always refer to the Safety Data Sheet for the most detailed information* |
| 3. NAME OF TRAINER / RESOURCE PERSON | Principal Investigator Name, Building, Room, Phone Number Secondary contact Name, Building, Room, Phone Number |
| - LOCATION OF HEALTH & SAFETY INFORMATION | The Safety Data Sheet (SDS) for 2-Methyl-2-butanol is located in the Laboratory Safety Manual in Building, Room. Labeling: Containers shall either have original warning label affixed or a label identifying the contents and hazards. |
| 5. PROTECTIVE EQUIPMENT | Wear chemical safety goggles, nitrile gloves, and a fully-buttoned lab coat. Wash hands after removing gloves. Work within a properly functioning, certified laboratory chemical fume hood. |
| - WASTE DISPOSAL PROCEDURES | Waste 2-Methyl-2-butanol is to be collected in a compatible waste container. A completed Dangerous Waste label should be attached when waste is first added to the container. When full the container must be accompanied by a completed Chemical Collection Request Form, and delivered to the Waste Accumulation Area Operator at Building, Room, Phone Number |
| 7. DESIGNATED AREA INFORMATION | The 2-Methyl-2-butanol is stored and dispensed in Building, Room. Work within a properly functioning certified laboratory chemical fume hood. The designated area(s) should be shown on the floor plan in Laboratories Chemical Hygiene Plan. |
| 8. DECONTAMINATION PROCEDURES | Upon Accidental Exposure: In case of eye contact, flush eyes with copious amounts of water at an emergency eyewash station for at least 15 minutes and consult a physician. In case of skin contact, flush skin with copious amounts of water for at least 15 minutes. Consult a physician. For exposure over a large portion of the body, remove clothing and shoes and rinse thoroughly in an emergency shower for at least 15 minutes. Consult a physician. In case of inhalation, move person to fresh air and consult a physician. In case of ingestion, immediately seek medical attention and follow instructions on SDS. Upon Accidental Release: Large Spill: If a large amount of 2-Methyl-2-butanol is released outside the fume hood, immediately evacuate, secure area and call 911 to contact EH&S. Small Spill: If a small amount of 2-Methyl-2-butanol is released (it can be cleaned up in 10 minutes) and you have been appropriately trained to clean it up, you may do so. Trained personnel should wear at the minimum nitrile gloves, chemical splash goggles, and a fully-buttoned lab coat. Additional PPE such as respirators may be necessary depending upon material and concentration. (Note: You must be medically cleared, fit tested and enrolled in WSU’s respiratory protection program to wear a respirator). If it is necessary to use a respirator and personnel are not cleared to wear a respirator and not trained to appropriately clean up the spill, the employee should immediately evacuate, secure area, and call 911 to contact EH&S. Absorb with an inert dry material and place in an appropriate waste disposal container and dispose of as dangerous waste (see above WASTE DISPOSAL PROCEDURES). As with all accidents, report any exposure as soon as possible to your Principal Investigator or Supervisor. Additional health and safety information on 2-Methyl-2-butanol can be obtained by referring to the SDS or by calling the EH&S Office (335-3041). |
| - SPECIAL STORAGE AND HANDLING PROCEDURES | Store in a cool, dry, well-ventilated area away from sunlight, heat, sparks, flame, or other sources of ignition. Store in a tightly closed container until ready for use. Keep away from incompatibles such as strong oxidizing agents and alkali metals. |

Certification of Hazard Assessment

Is this document a certification of Hazard Assessment for the processes identified within?                            			Yes					No

If yes, provide the name of the person certifying the Hazard Assessment and the date it was performed:

____________________________________________________________________________________

Name									Date

The location of the Hazard Assessment is indicated in the document preceding this form.

Certificate of Employee Training

Name of person providing training for employees working with this process:

___________________________________________________________________________________

The following employees have been trained in when, where and how to use selected PPE, the maintenance, limitations and disposal of the PPE selected, and have demonstrated the correct use of the PPE selected on the reverse of this certification.

Name								Date Trained

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________

________________________________________                         _______________________________